IPC Production Tool User Manual

- 1. Tool design instructions

- The main purpose of this tool is to quickly test the IPC motherboard and complete the customer's default customization;

- 2.Tool testing process

As shown in the figure, please pay special attention to the red mark, otherwise it may cause invalid problems such as setting items

- Test flow chart 1

- 3. Setting item description

- 3.1 Description of test items:

- Language detection, standard detection Check the factory language and standard of the version

- Version detection is divided into version information, system time and version cloud verification. In view of the confusion of TS cloud and LT cloud on Ruijie's side, it is prompted by checking whether the ID (beginning with rj and jr) and the version match. System time detection is to detect whether the IPC time is consistent with the computer time. There are three main states of cloud verification: success, failure, and unrecognized. Success is displayed in green, and the remaining two items are displayed in red.

- WIFI detection: identify by reading whether there is a WIFI module

- Test audio frequency: temporarily not supported

- SD detection: Recognize by reading whether there is an SD card

- MAC address detection: check the first 6 digits of the MAC address

- 3.2 Setting item description

- Language and standard: Set the language and standard to facilitate production.

- Sync time: synchronize the device time with the computer time.

- Enable audio encoding: enable IPC audio encoding.

- Set audio input: currently only supports Line in.

- Set LocalNo, lens flip, mirror, IR reverse sequence, set time zone: set these functions to facilitate production.

- Light board mode: divided into infrared mode, full color mode and dual light source mode.

- Dual light source mode is divided into two photosensitive modes: soft photosensitive and hard photosensitive.

- Dual light source mode has three kinds of night vision: infrared night vision, full-color night vision and dual-light night vision.

- Restore default: do not restore language and format, and light board settings

- SuperR: If the IP is not restored, other settings will be restored

- 4.Manual test item description

- ：Turn on intercom

- ：Stop intercom

- ：Turn on sound

- ：Stop sound

- ：Turn on the warm light

- ：Turn off the warm light